Train and Go Operating Procedure

Garrett Bradshaw, Slade Hicks, Brandon Waldrup, and Kyler Smith

Purpose

Train and go is a training device designed to help users master the skills required to maneuver a wheelchair. This Device uses virtual reality simulation coupled with a separate operation detection system to help the user achieve this goal. Below is a detailed guide of how to install/operate Train and Go.

Key Terms/Definitions

Virtual Reality: A virtual environment uploaded to the Meta Quest Pro headset.

Ultrasonic Sensors: Sensors to be placed along the base of the chair to detect obstacles.

Microcontroller: An integrated circuit device that governs a specific operation in an embedded system.

Printed Circuit Board (PCB): Circuit board with all the necessary connections for the microcontroller and ultrasonic sensors to operate.

Inertial Measurement Unit (IMU): A device that can measure and report specific gravity and angular rate of an object to which it is attached.

Rumble Motor: Mechanical device that generates vibrations.

Installation

- Using the Velcro straps, secure the housing box containing the microcontroller, PCB, and battery to the back of the chair.

  - This location should allow easy charging access for the battery.

- Using the Velcro Straps, secure the Meta Quest Pro controller mount to the back of the head rest of the chair.

- Using the provided nuts, secure the ultrasonic sensor mounts along the rail on the sides of the chair.

Forward

Adjacent

Backward

There are 6 sensors and 6 sensor mounts. Each mount is designed to face a specific direction (Forward, Adjacent, Backward) on each side of the chair.

- Place the ultrasonic sensors in each sensor mount.

- Connect each wire from the housing box to the corresponding sensor.

  - Wires can be secured to the siding of the chair.

- Secure rumble motor inside the back cushion.

  - This should be secured in the upper portion of the chair to ensure the user will feel the intended vibration.

  - Wiring can be secured to the chair once the rumble motor is set in place.

- Make sure all connections are secured and flip the switch on the housing box.

  - Test that each sensor is properly triggering the rumble motor to vibrate at the specified distances in the table below.

| Sensor | Detection Range Start (mm) | Detection Range End (mm) |
| --- | --- | --- |
| Forward-Left | 10 | 20 |
| Adjacent-Left | 10 | 20 |
| Backward-Left | 10 | 20 |
| Forward-Right | 10 | 20 |
| Adjacent-Right | 10 | 20 |
| Backward-Right | 10 | 20 |

Virtual Reality Installation/Setup

Sideload the file TrainandGo.apk onto a VR headset. It is designed for use on Meta Quest Pro.
Sideloading instructions for a Meta Quest device are as follows:

- Install SideQuest or any sideloading software to a source computer.

  - This computer does not need to be powerful enough for VR: it will just transfer files.

  - See https://sidequestvr.com/setup-howto for installation instructions. Use the “Advanced Installer.”

- Enable developer mode on Meta Quest device.
https://help.arborxr.com/en/articles/6333136-developer-mode-usb-debugging-on-meta-quest-devices

- Connect Meta Quest Pro to the source computer.

- Download TrainandGo.apk to the source computer.

- Load TrainandGo.apk onto the Meta Quest device using SideQuest.

  - Drag TrainandGo.apk onto the green bubble in the SideQuest window.

- When SideQuest informs you that the transfer is complete, disconnect the headset.

The remaining VR Instructions are as follows:

- Using the VR Headset, map the boundaries of the room.

- Place the right-hand VR controller in the controller mount that was installed on the back of the wheelchair.

- Use the left-hand VR controller to navigate to the Unknown Sources section of the Apps menu.

- Open TrainandGo.apk.

- Press the trigger on the previously mounted right-hand controller to ensure the controller is “awake” and track the wheelchair movement properly.

- (Optional) Screen share the headset display to a screen for viewing.

  - This can be done through SideQuest or Meta’s screen share service. See Links Below: